ICD-106: Payload EMI/EMC Interface

1. Document Control

- Document Number: SDSU-SB+-ICD-106

- Revision: v1.0

- Date: March 2026

- Owner: SDSU Payload Team

2. Purpose

This document defines the EMI/EMC interface of SDSU payload subsystems, including RILA, ZES, and supporting sensors. It defines electrical noise, grounding, shielding, and susceptibility assumptions.

3. Scope

This ICD governs:

- Establish electrical noise assumptions

- Grounding assumptions

- Shielding assumptions

- Susceptibility assumptions

4. Electrical Noise/ EMI/EMC

- MS1002 Accelerometer:

  - Accelerometers will be provided in an antistatic bag with ESD warning labels on it.

  - Only remove the accelerometer from the packaging when it is being used.

  - Manipulate the device within an ESD-controlled environment.

  - Store devices in a shielded environment that protects against ESD damage.

  - Wear a wrist strap and use ESD-safe gloves when handling the device.

- Rila:

  - N/A

- ZES:

  - N/A

- Magnetometer:

  - Will need to be distanced from electronic components like motors, high‑current lines, reaction wheels, batteries, and deployable solar panels to improve measurement accuracy.

5. Grounding/Shielding/Susceptibility

- MS1002 Accelerometer:

  - The sensor is sensitive to external parasitic capacitance, thus a ground plane directly below the accelerometer is recommended for shielding purposes.

- Rila:

  - RS-422 cabling shall use twisted shielded pairs (100–120 Ω).

  - Shield termination only at ZES end to prevent ground loops.

- ZES:

  - 2mm shielding of aluminum will allow for an adequate TID lifespan.

- Magnetometer:

  - No shielding is required