Interface Control Document (ICD): Payload to Bus

Revision: B
 Date: 2025-11-17
 Prepared By: Communications/Software Lead (Avrey)
 Reviewed By:

### 1. Interface Overview

This ICD defines the electrical, mechanical, thermal, and data interfaces between the Payload Subsystem (RILA dosimeter, ZES board, magnetometer, and accelerometer) and the On-Board Computer (OBC) of the Sentinel Bio+ CubeSat. The document ensures proper physical mounting, power compatibility, signal format, and data flow between the payload and the bus.

### 2. Subsystem Overview

| Component / Script Name | Description | Primary Function |
| --- | --- | --- |
| RILA Dosimeter | Radiation dosimeter with onboard memory | Capture radiation exposure readings |
| ZES Board | Electronics support board | Interfaces RILA and sensors to OBC |
| Magnetometer | Magnetic field sensing | Part of science data payload |
| Accelerometer | Movement/orientation sensing | Science data for bio-exposure modeling |
| OBC Interface Board | Interface connector to CubeSat bus | Provides power and command/data handling |

### 3. Electrical Interfaces

| ### Component | ### Power Requirement | ### Connector Type | ### OBC Supply Voltage | ### Current Draw |
| --- | --- | --- | --- | --- |
| ### RILA Dosimeter | ### 12 V ±0.5 V | ### DB-9 | ### 12V | ### 30 mA nominal, 42 mA peak |
| ### ZES Board | ### 5 V ±5 % (3.3 V logic) | ### 49-pin Header | ### 5V | ### 120–200 mA nominal, ≤500 mA max |
| ### Sensors Magnetometer | ### 3.3V 3.3V ⨦TBD | ### I²C header RS-422 UART | ### 3.3V 3.3V ⨦TBD | ### <20 mA combined ———— |

### 4. Data Interfaces

| ### Interface | ### Protocol | ### Baud Rate | ### Data Type | ### Direction | ### Notes |
| --- | --- | --- | --- | --- | --- |
| ### RILA ↔ ZES | ### RS-422 (via converter) | ### 115200 bps | ### 522-byte spectrum frames | ### Bidirectional | ### Periodic radiation dump |
| ### ZES ↔ OBC | ### UART (3.3 V) | ### 115,200 bps | ### Mode A: 522 B full frame, Mode B: ≤340 B summary | ### Bidirectional | ### OBC polls, ZES replies |
| ### Sensors (Mag/Accel) | ### I²C/SPI/ ### UART | ### TBD | ### 3-axis data | ### ZES→OBC |  |
| ### Payload Sync Signal | ### GPIO / PWM | ### N/A | ### Trigger / status line | ### Bidirectional | ### For fail-safe or sync function |

### 5. Mechanical and Structural Interfaces

- Payload components mounted on Bus Payload Plate (Deck 2 of 3U structure)

- Isolated with vibration damping standoffs

- Must comply with CubeSat form factor restrictions (100 mm x 100 mm per U)

- Center of mass kept aligned with structural centroid

- All connectors shall be accessible for integration and test

- RILA 100 × 78 × 27 mm, 4×M3 holes (approx 62 × 90 mm pattern)*final mount hole pattern TBD.

- DB-9 connector front-mounted with 13.5 mm offset.

- ZES 70×50 mm PCB on M2.5 standoffs.

- Shared mounting deck with thermal contact ≥ 80 %.

- Magnetometer keep-out ≥ 100 mm from ferrous materials.

### 6. SWaP

| Component | Mass (kg) | Margined mass (kg) |
| --- | --- | --- |
| RILA Dosimeter | < 0.200 | ————————— |
| ZES Magenetometer ADXL355 Accelerometer MS1002 Accelerometer | 0.013 0.04 0.00026 0.0015 | ————————— ————————— ————————— ————————— |

| Component | Volume (mm3) |
| --- | --- |
| RILA Dosimeter | 210600 |
| ZES Magenetometer ADXL355 Accelerometer MS1002 Accelerometer Total 3U | 24300 300000 73.92 256.6404 616230.56 3000000 |

Based on the rough estimates of the volumes for the payload, the space required for the 3U satellite is roughly 0.2U but should be around 1.0U for spacing purposes. We are aiming to keep the payload volume to less than 1.5U, and believe for adequate spacing between payload sub-systems we need roughly 1.0U of space.

| Component | Nominal Power (W) | Peak Power (W) |
| --- | --- | --- |
| RILA Dosimeter | 0.35 | 0.50 |
| ZES Magenetometer ADXL355 Accelerometer MS1002 Accelerometer | 0.60 - 1.00 0.065 + TBD RS-442 Overhead <0.01 0.0132 | 2.50 Awaiting info. from vendor <0.01 0.0136 |

### 7. Thermal Interface

| Component | Operating Temp Range | Thermal Interface Method |
| --- | --- | --- |
| RILA Dosimeter | −20 °C to +40 °C (operational), −40 °C to +75 °C (survival) | Conductive coupling (aluminum base, thermal pads) |
| ZES + Sensors Magnetometer ADXL 355 Accelerometer MS1002 Accelerometer | −40 °C to +105 °C -30 °C to +70 °C −40 °C to +125 °C −40 °C to +125 °C | Conductive via standoffs and PCB copper planes TBD by vendor Conductive via standoffs and PCB planes Conductive via standoffs and PCB planes |

### 8. ZES Operation Data

| Description | Value/Expectation |
| --- | --- |
| Idle current draw at 5 V (MCU powered, minimal activity) | (Equipment is not accurate enough to determine less than 500 mA. Will need to retest with new equipment to get a more accurate value) |
| Active/processing current draw at 5 V (FRAM writes, UART activity, SPI/I2C activity) Peak/transient current during CPU or I/O spikes Expected Duty cycle for typical operation (active vs. idle percentage). Addition margin or derating recommendation | 12 - 16 mA (Based on our starter program) 22 mA (Based on Coremark Program) Depends upon users program 25 - 30% |

Figure 1. Flow Chart between ZSOM board and OBC

### 9. Data Handling/Software Hooks/Data Management

- Payload data is queued in a shared OBC file buffer

- File naming convention: YYYYMMDD_payload.bin

- OBC reads and schedules for S-Band transmission

- Fault flag or unusual readings will trigger Iridium beacon

- The data sent out by our payload, when requested every 15 mins, are formatted as table below and the total size will be 6242 bytes. Through this inference the size of the data per day will be around 599232 bytes.

Table 1. Download Data Format

| S/N | Data | Bytes |
| --- | --- | --- |
| 1 | Header (0x2A) | 1 |
| 2 | Data | 6240 |
| 3 | Tail (0xAA) | 1 |
|  | Total | 6242 |

Table 2. Data Interpretation of the Downlink Data (per set)

Figure 1. I2C Timing Diagram — Single-Byte Read

Figure 2. I2C Timing Diagram — Single-Byte Write

Figure 3. I2C Timing Diagram — Multi-Byte Write

Will add magnetometer data handling for and downlink once the vendor has available information.

### 10. Verification and Test

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Electrical Power Test | Supply voltage + current log | 3.3V within 5%; < 300 mA draw |
| UART Data Transfer | Serial packet logging | No packet loss / corruption over 1hr |
| Mechanical Fit Check | CAD model + prototype | No interference or blocking |
| Thermal Profiling | Chamber or passive test | Stays within specified thermal range |
| RS-422 ↔ UART Link | Serial monitor log | ≥99.9 % valid CRC frames |
| RILA Power Line I2C Links | Measure voltage/current Serial monitor log | 12 V ±0.5 V, <0.5 W peak ≥99.9 % valid response frames from accelerometers |

### 9. Signature Block and Revision Control

| Team | Name | Signature | Date |
| --- | --- | --- | --- |
| SDSU | Lujain | __________________ | 2025-11-23 |
| SDSI | [Insert Name] | __________________ | 2025-MM-DD |
| SJSU | [Insert Name] | __________________ | 2025-MM-DD |

| Rev. | Date | Description | Editor |
| --- | --- | --- | --- |
| A B C | 2025-11-17 2025-12-15 2026-03-12 | Initial Draft 1st Revision 2nd Revision | Lujain Jacob Ogle Jacob Ogle |